This target best for a 2 MOA red dot scope at distances from 50 to 70 yards

# Directions For Use

- Set target at the near zero range.

For a 275 yard far zero

M4 M855 - zero at 36.5 yards

M16A1/A2 M193 –zero at 41 yards			16” M4 M855 - zero at 37 yards

M16A2 M855 –zero at 39 yards				16” M4 M193 –zeroat 42 yards

Or use a ballistics program to determine your particular near zero range

2) Use round label “dots” for the aiming point, available from OnlineLabels.com

For a 2 moa red dot scope use the .75” (Onlinelabels.com # OL5275WX), the 1” (Onlinelabels.com # OL1025WX), or the 1.2” (Onlinelabels.com # OL6000WX) dots and center the dot on the black field on the reverse.

For a 4 moa red dot scope use the .1.5” (Onlinelabels.com # OL2088WX0, the 1.67” (Onlinelabels.com # OL325WX), or the2” (Onlinelabels.com# OL5375WX) dots and center the dot on the black field on the reverse

3)  Align your red dot so there is an even band of white showing around the red dot and fire.